Practical 5 - Firewall Configuration and Rule-based Filtering

Aim: Configure and test firewall rules to control network traffic, filter packets based on

specified criteria, and protect network resources from unauthorized access.

### Topology

### Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway | Switch Port |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/1 | 192.168.1.1 | 255.255.255.0 | N/A | S1 F0/5 |
|  | S0/0/0 (DCE) | 10.1.1.1 | 255.255.255.252 | N/A | N/A |
| R2 | S0/0/0 | 10.1.1.2 | 255.255.255.252 | N/A | N/A |
|  | S0/0/1 (DCE) | 10.2.2.2 | 255.255.255.252 | N/A | N/A |
| R3 | G0/1 | 192.168.3.1 | 255.255.255.0 | N/A | S3 F0/5 |
|  | S0/0/1 | 10.2.2.1 | 255.255.255.252 | N/A | N/A |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 | S1 F0/6 |
| PC-C | NIC | 192.168.3.3 | 255.255.255.0 | 192.168.3.1 | S3 F0/18 |

Objectives

- Verify connectivity among devices before firewall configuration.

- Configure a zone-based policy (ZPF) firewall on R3.

- Verify ZPF firewall functionality using ping, SSH, and a web browser.

### Background/Scenario

ZPFs are the latest development in the evolution of Cisco firewall technologies. In this activity, you will configure a basic ZPF on an edge router R3 that allows internal hosts access to external resources and blocks external hosts from accessing internal resources. You will then verify firewall functionality from internal and external hosts.

The routers have been pre-configured with the following:

- Console password: ciscoconpa55

- Password for vty lines: ciscovtypa55

- Enable password: ciscoenpa55

- Host names and IP addressing

- Local username and password: Admin / Adminpa55

- Static routing

For all routers

Rl(config)#line c 0

Rl(config-line)#password ciscoconpa55

Rl(config-line)#exit

Rl(config)#line vty O 4

Rl(config-line)#password ciscovtypa55

Rl(config-line)#exit

Rl(config)#enable password ciscoenpa55

Rl(config)#username Adrnin password Adrninpa55

Static Routing For Rl

Static Routing For R2

Static Routing For R3

Part 1: Verify Basic Network Connectivity

Verify network connectivity prior to configuring the zone-based policy firewall.

Step 1: From the PC-A command prompt, ping PC-C at 192.168.3.3. Step 2: Access R2 using SSH.

- From the PC-C command prompt, SSH to the S0/0/1 interface on R2 at 10.2.2.2. Use the username

Admin and password Adminpa55 to log in.

To Enable SSH in Router 2

R2(config)#line vty O 4 R2(config-line)#login local

R2(config-line)#transport input ssh R2(config-line)#exit

R2(config)#ip domain-name ccna.com R2(config)#crypto key generate rsa

The name for the keys will be: R2.ccna.com

Choose the size of the key modulus in the range of 360 to 2048 for your General-Purpose Keys. Choosing a key modulus greater than 512 may take a few minutes.

How many bits in the modulus [512]:

% Generating 1024 bit RSA keys, keys will be non-exportable...[OK]

From PC Command Prompt

PC> ssh -1 Adrnin 10.2.2.2

- Exit the SSH session.

#### Step 3: From PC-C, open a web browser to the PC-A server.

- Click the Desktop tab and then click the Web Browser application. Enter the PC-A IP address

192.168.1.3 as the URL. The Packet Tracer welcome page from the web server should be displayed.

- Close the browser on PC-C.

## Part 2: Create the Firewall Zones on R3

Note: For all configuration tasks, be sure to use the exact names as specified.

#### Step 1: Enable the Security Technology package.

- On R3, issue the show version command to view the Technology Package license information.

- If the Security Technology package has not been enabled, use the following command to enable the package.

R3(config)# license boot module cl900 technology-package securityk9 ACCEPT? [yes/no]: y

Rl(config)#do write

Rl(config)#do reload

Rl#Show Version

- Accept the end-user license agreement.

- Save the running-config and reload the router to enable the security license.

- Verify that the Security Technology package has been enabled by using the show versioncommand.

#### Step 2: Create an internal zone.

#### Use the zone security command to create a zone named IN-ZONE.

R3(config)# zone security IN-ZONE R3(config-sec-zone) exit

#### Step 3: Create an external zone.

Use the zone security command to create a zone named OUT-ZONE.

R3(config)# zone security OUT-ZONE R3(config-sec-zone)# exit

## Part 3: Identify Traffic Using a Class-Map

#### Step 1: Create an ACL that defines internal traffic.

Use the access-list command to create extended ACL 101 to permit all IP protocols from the 192.168.3.0/24

source network to any destination.

R3(config)# access-list 101 permit ip 192.168.3.0 0.0.0.255 any

#### Step 2: Create a class map referencing the internal traffic ACL.

Use the class-map type inspect command with the match-all option to create a class map named IN-NET-CLASS-MAP. Use the match access-group command to match ACL 101.

R3(config)# class-map type inspect match-all IN-NET-CLASS-MAP R3(config-cmap)# match access-group 101

R3(config-cmap)# exit

## Part 4: Specify Firewall Policies

#### Step 1: Create a policy map to determine what to do with matched traffic.

Use the policy-map type inspect command and create a policy map named IN-2-OUT-PMAP.

R3(config)# policy-map type inspect IN-2-OUT-PMAP

#### Step 2: Specify a class type of inspect and reference class map IN-NET-CLASS-MAP.

R3(config-pmap)# class type inspect IN-NET-CLASS-MAP

#### Step 3: Specify the action of inspect for this policy map.

The use of the inspect command invokes context-based access control (other options include pass and drop).

R3(config-pmap-c)# inspect

%No specific protocol configured in class IN-NET-CLASS-MAP for inspection. All protocols will be inspected.

Issue the exit command twice to leave config-pmap-c mode and return to config mode.

R3(config-pmap-c)# exit R3(config-pmap)# exit

## Part 5: Apply Firewall Policies

#### Step 1: Create a pair of zones.

Using the zone-pair security command, create a zone pair named IN-2-OUT-ZPAIR. Specify the source and destination zones that were created in Task 1.

R3(config)# zone-pair security IN-2-OUT-ZPAIR source IN-ZONE destination OUT- ZONE

#### Step 2: Specify the policy map for handling the traffic between the two zones.

Attach a policy-map and its associated actions to the zone pair using the service-policy type inspect

command and reference the policy map previously created, IN-2-OUT-PMAP.

R3(config-sec-zone-pair)# service-policy type inspect IN-2-OUT-PMAP R3(config-sec-zone-pair)# exit

R3(config)#

#### Step 3: Assign interfaces to the appropriate security zones.

Use the zone-member security command in interface configuration mode to assign G0/1 to IN-ZONE and S0/0/1 to OUT-ZONE.

R3(config)# interface g0/1

R3(config-if)# zone-member security IN-ZONE R3(config-if)# exit

R3(config)# interface s0/0/1

R3(config-if)# zone-member security OUT-ZONE R3(config-if)# exit

Step 4: Copy the running configuration to the startup configuration.

## Part 6: Test Firewall Functionality from IN-ZONE to OUT-ZONE

Verify that internal hosts can still access external resources after configuring the ZPF.

#### Step 1: From internal PC-C, ping the external PC-A server.

From the PC-C command prompt, ping PC-A at 192.168.1.3. The ping should succeed.

#### Step 2: From internal PC-C, SSH to the R2 50/0/1 interface.

- From the PC-C command prompt, SSH to R2 at 10.2.2.2. Use the username Admin and the password

Adminpa55 to access R2. The SSH session should succeed.

- While the SSH session is active, issue the command show policy-map type inspectzone-pair sessions on R3 to view established sessions.

#### R3# show policy-map type inspect zone-pair sessions

policy exists on zp IN-2-OUT-ZPAIR Zone-pair: IN-2-OUT-ZPAIR

Service-policy inspect : IN-2-OUT-PMAP

Class-map: IN-NET-CLASS-MAP (match-all) Match: access-group 101

Inspect

Number of Established Sessions= 1 Established Sessions

Session 175216232 (192.168.3.3:1028)=>(10.2.2.2:22) tcp SIS_OPEN/TCP_ESTAB

Created 00:00:25, Last heard 00:00:20  Bytes sent (initiator:responder) [1195:1256] Class-map: class-default (match-any) Match:

Drop (default action)

0 packets, 0 bytes

What is the source IP address and port number?

192.168.3.3: 1028 (port 1028 is random)

What is the destination IP address and port number?

10.2.2.2:22 (SSH = port 22)

#### Step 3: From PC-C, exit the SSH session on R2 and close the command prompt window. Step 4: From internal PC-C, open a web browser to the PC-A server web page.

Enter the server IP address 192.168.1.3 in the browser URL field, and click Go. The HTTP session should succeed. While the HTTP session is active, issue the command show policy-map type inspect zone-pair sessions on R3 to view established sessions.

Note: If the HTTP session times out before you execute the command on R3, you will have to click the Go

button on PC-C to generate a session between PC-C and PC-A.

R3# show policy-map type inspect zone-pair sessions

policy exists on zp IN-2-OUT-ZPAIR Zone-pair: IN-2-OUT-ZPAIR

Service-policy inspect : IN-2-OUT-PMAP

Class-map: IN-NET-CLASS-MAP (match-all) Match: access-group 101

Inspect

Number of Established Sessions= 1 Established Sessions

Session 565266624 (192.168.3.3:1031)=>(192.168.1.3:80) tcp SIS_OPEN/TCP_ESTAB

Created 00:00:01, Last heard 00:00:01

sent (initiator:responder) [284:552]

Class-map: class-default (match-any)

Match: any

)rop (default action)  packets, 0 bytes

What is the source IP address and port number?

192.168.3.3: 1031 (port 1031 is random) What is the destination IP address and port number?

192.168.1.3:80 (HTTP web = port 80)

#### Step 5: Close the browser on PC-C.

## Part 7: Test Firewall Functionality from OUT-ZONE to IN-ZONE

Verify that external hosts CANNOT access internal resources after configuring the ZPF.

#### Step 1: From the PC-A server command prompt, ping PC-C.

From the PC-A command prompt, ping PC-C at 192.168.3.3. The ping should fail.

#### Step 2: From R2, ping PC-C.

From R2, ping PC-C at 192.168.3.3. The ping should fail.